AAP Receiver Program 1.0.2.8

The Receiver program is configured from the SETUP form.

Page limit:

This entry defines the number of lines retained in the display window.

TCP Port:

The Port number the program will accept TCP data on.

UDP Port:

The Port number the program will accept UDP data on.

COM Port:

Select the Serial Port to be used to communicate with the Monitoring Station Software.

ReceiverID:

Set this field to the assigned Receiver ID. This ID would be supplied by the Monitoring Station.

Line ID:

Set this field to the assigned Line ID. This ID would be supplied by the Monitoring Station.

Poll Timer:

In TCP Mode, if the Receiver Program does not receive an acknowledgement from the Monitoring Station, the event is acknowledged to the panel and queued for later transmission to the Monitoring station. After 'Poll Timer' seconds the Receiver will attempt to resend the event to the Monitoring Station.

In UDP Mode, the Receiver program will only acknowledge the event to the panel on receipt of an acknowledgement from the Monitoring station. This field is ignored in this mode..

Local Monitor Station

Enabling this will create a Receiver which simply ECHOES the received message to the panel. If CSV, Patriot LS-30 or AAP Encrypted are selected on the IP-Module, the appropriate ACK is returned to the panel.

If there is a valid ComPort, then the module will expect input from the ComPort in RS232 Serial CID Format and respond with ACK (emulating the Monitoring Software)

eg <0A>01 3375 18 E408 01 U000 <0D>

Poll:

If this is checked then the Receiver program will monitor the Com Port for the HEX String defined by Poll. In this example the letter 'A'.

If received, the Poll Resp HEX String will be sent back to the Monitoring Station. In this example the string 'ACK'.

Protocol

The receiver program will automatically accept:

CSV IP Format

PATRIOT LS-30

AAP ECID (RSA RC4 encrypted with random Key)

All output to the Monitoring Software is in the selected Receiver Format (SURGARD or ADEMCO)

Log:

The Receiver Program will generate a log entry for each record acknowledged by the Monitoring Station.

These records are found in the LOGS sub directory of the working Directory.

The file name is YYYYMMDD-HH-LL.LOG. Eg 20130618-08-01.log

08 being the hour

01 being the line number

An example of the log entries:

08:42:13 : [10.254.0.220] <0A>01 3375 18 E408 01 U000 <0D>

08:42:16 : [10.254.0.220] <0A>01 3375 18 R408 01 U000 <0D>